МОСКОВСКИЙ АВИАЦИОННЫЙ ИНСТИТУТ

(НАЦИОНАЛЬНЫЙ ИССЛЕДОВАТЕЛЬСКИЙ УНИВЕРСИТЕТ)

Институт №8 «Компьютерные науки и прикладная математика»

Кафедра №806 «Вычислительная математика и программирование»

Лабораторная работа №2

по курсу «Вычислительные сети и телекоммуникации»

Настройка L2-коммутаторов с избыточными

подключениями и VLAN

Выполнил: Почечура А.А.

Группа: 8О-406Б

Преподаватель: Филимонов Н.С.

Москва, 2024

Условие

Цель работы. Научиться конфигурировать многоуровневые коммутаторы L2/L3 с VLAN, RSTP, LACP, использовать inter-VLAN routing, настраивать статическую маршрутизацию и конечный пользовательский NAT на Cisco.

Вариант 18.

Задачи

- Настроить имена узлов для коммутаторов и маршрутизатора в соответствии с именами, указанными в топологии.

- Сменить пароль администратора (пароль доступа к привилегированному режиму enable для Cisco) на приведенный в Вашем варианте ЛР для соответствующего устройства.

- Создать пользователя checker с максимальным административным уровнем доступа и паролем “PfxtvXtrth!” без кавычек на коммутаторах и маршрутизаторе.

- Настроить на маршрутизаторе R1 IP-адреса интерфейсов в соответствии с вариантом задания.

- Настроить на маршрутизаторе R1 встроенный DNS-сервер с передачей запросов на сервер QuadNine (9.9.9.9), включить удаленные запросы.

- Переключить интерфейсы коммутаторов опорной сети в режим trunk 802.1q за исключением access-портов (GigabitEthernet1/0) на SW4 и SW5.

- Организовать на коммутаторах SW1, SW2, SW3 группы портов между соседними коммутаторами в кольце в каналы EtherChannel в режиме LACP active, перевести каналы EtherChannel в транк 802.1q.

- Настроить VLANы на коммутаторах в соответствии с таблицей конфигурации в Вашем варианте вручную или (*) с помощью VTP.

- Настроить STP на коммутаторах таким образом, чтобы для всех VLAN SW1 являлся root bridge, а на коммутаторах SW4 и SW5 root-портами являлись интерфейсы GigabitEthernet0/0.

- Разрешить на всех транковых портах коммутаторов только VLAN из таблицы в явном виде. На интерфейсе GigabitEthernet0/0 на коммутаторе SW1 (в сторону маршрутизатора R1) разрешить только VLAN Internet и VLAN Management.

- Настроить на коммутаторе SW1 IP-адрес на интерфейсе VLAN Internet в соответствии с вариантом задания, адрес DNS-сервера (адрес маршрутизатора R1) и маршрут по умолчанию на маршрутизатор R1.

- На коммутаторе SW1 назначить IP-адреса во всех VLAN и включить ip routing.

- Назначить IP-адреса в Management VLAN на всех коммутаторах согласно варианту и установить маршрут по умолчанию на адрес коммутатора SW1.

- Настроить на всех коммутаторах в качестве DNS-сервера адрес маршрутизатора R1.

- Настроить на маршрутизаторе R1 статические маршруты на сети из таблицы VLAN, ведущие на коммутатор ядра SW1.

- Настроить на маршрутизаторе R1 трансляцию адресов (NAT) для доступа в интернет IP-адресов из VLAN Internet, используя в качестве глобального адреса адрес на интерфейсе GigabitEthernet0/0.

- Настроить на коммутаторе SW1 DHCP-сервер во VLAN Internet и Cash, используя пулы адресов с маской /24, маршрут по умолчанию на SW1 и DNS-сервер на R1. Назначить исключения в пулах для статических адресов, назначенных сетевым устройствам.

- Перевести порты GigabitEthernet1/0 на коммутаторах SW4 и SW5 в режим access и назначить им VLAN Internet (SW4) и Cash (SW5) соответственно. Проверить доступ в Интернет с узла Win (Windows 10) в режиме DHCP.

- Настроить сервер Server под управлением Ubuntu 20.04 интерфейс eth0 в режиме trunk 802.1Q со всеми VLAN из таблицы, используя netplan и статические адреса из Вашего варианта. Проверить доступ в Интернет с сервера, а также доступ к серверу с узла Win и узла Linux.

Порядок выполнения работы

- Настройка имён узлов в соответствии с именами, указанными в топологии осуществляется следующим образом:

Для маршрутизатора:

enable

conf t

hostname R1

Для коммутаторов:

enable

conf t

hostname SWN (N – номер коммутатора в топологии)

- Смена пароля администратора (пароль доступа к привилегированному режиму enable для Cisco) ЛР для соответствующего устройства производится следующим образом:

aaa new-model

service password-encryption

enable secret admin18

- Создание пользователя checker с максимальным административным уровнем доступа и паролем “PfxtvXtrth!” без кавычек на коммутаторах и маршрутизаторе осуществляется следующим образом:

username checker privilege 15 password 0 PfxtvXtrth!

- Настройка на маршрутизаторе R1 IP-адреса интерфейсов производится следующим образом:

interface gigabitEthernet 0/0

no shutdown

(exit)

interface gigabitEthernet 0/1

no shutdown

interface gigabitEthernet 0/1.218

encapsulation dot1Q 218

ip address 192.168.218.1 255.255.255.0

no shutdown

interface gigabitEthernet 0/1.118

encapsulation dot1Q 118

ip address 192.168.118.1 255.255.255.0

no shutdown

- Настройка на маршрутизаторе R1 встроенного DNS-сервера с передачей запросов на сервер QuadNine (9.9.9.9) и включение удаленных запросов осуществляется следующим образом:

ip domain-lookup

ip dns server

ip name-server 9.9.9.9

- Переключение интерфейсов коммутаторов опорной сети в режим trunk 802.1q за исключением access-портов (GigabitEthernet1/0) на SW4 и SW5 производится следующим образом:

Для SW2 и SW3:

interface range gi0/1,gi0/0

switchport trunk encapsulation dot1q

switchport mode trunk

На SW4 и SW5 это происходит также, с заменой соответствующих интерфейсов.

- Организация на коммутаторах SW1, SW2, SW3 группы портов между соседними коммутаторами в кольце в каналы EtherChannel в режиме LACP active, перевод каналов EtherChannel в транк 802.1q осуществляется следующим образом:

На SW1:

interface Port-Channel 1

(exit)

interface range  gi0/1,gi0/2

channel-group 1 mode active

(exit/end)

interface Port-Channel 2

(exit)

interface range  gi1/1,gi1/0

channel-group 2 mode active

(exit/end)

interface Port-Channel 1

switchport trunk encapsulation dot1q

switchport mode trunk

(exit/end)

interface Port-Channel 2

switchport trunk encapsulation dot1q

switchport mode trunk

(exit/end)

На SW2 и SW3 это происходит также, с заменой соответствующих интерфейсов.

- Настройка VLANов на коммутаторах в соответствии с таблицей конфигурации с помощью VTP производится следующим образом:

На SW1:

vtp mode server

vtp domain LABAZ

vtp version 3

(exit)

vtp primary vlan

vlan 1N

name MGMT

vlan 2N

name Internet

vlan 3N

name Cash

На остальных коммутаторах:

vtp domain LABAZ

vtp version 3

vtp mode client

- Настройка STP на коммутаторах таким образом, чтобы для всех VLAN SW1 являлся root bridge, а на коммутаторах SW4 и SW5 root-портами являлись интерфейсы GigabitEthernet0/0 осуществляется следующим образом:

На SW1:

spanning-tree vlan 118,218,318 root primary

На SW4 и SW5:

interface gi0/0

spanning-tree cost 3

- Разрешение на всех транковых портах коммутаторов только VLAN из таблицы в явном виде, а на интерфейсе GigabitEthernet0/0 на коммутаторе SW1 (в сторону маршрутизатора R1) разрешение только VLAN Internet и VLAN Management производится следующим образом:

На SW1:

interface gi0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 118, 218

interface Port-Channel 1

switchport trunk allowed vlan 118,218,318

interface Port-Channel 2

switchport trunk allowed vlan 118,218,318

interface gi0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 118, 218, 318

На SW2 и SW3:

interface Port-Channel 1

switchport trunk allowed vlan 118,218,318

interface Port-Channel 2

switchport trunk allowed vlan 118,218,318

interface range gi1/0,gi1/1

switchport trunk allowed vlan 118,218,318

На SW4 и SW5:

interface range gi0/0,gi0/1

switchport trunk allowed vlan 118,218,318

- Настройка на коммутаторе SW1 IP-адрес на интерфейсе VLAN Internet, адрес DNS-сервера (адрес маршрутизатора R1) и маршрут по умолчанию на маршрутизатор R1 осуществляется следующим образом:

interface vlan 218

ip address 192.168.218.201 255.255.255.0

no shutdown

ip domain-lookup

ip name-server 192.168.218.1

ip route 0.0.0.0 0.0.0.0 vlan218 192.168.218.1

- Назначение на коммутаторе SW1 IP-адреса во всех VLAN и включение ip routing производится следующим образом:

interface vlan 118

ip address 192.168.118.201 255.255.255.0

no shutdown

interface vlan 318

ip address 192.168.18.201 255.255.255.0

no shutdown

- Назначение IP-адреса в Management VLAN на всех коммутаторах и установка маршрута по умолчанию на адрес коммутатора SW1 осуществляется следующим образом:

interface vlan 118

ip address 192.168.118.20N 255.255.255.0  (N – номер коммутатора)

no shutdown

ip route 0.0.0.0 0.0.0.0 vlan118 192.168.118.201

- Настройка на всех коммутаторах в качестве DNS-сервера адрес маршрутизатора R1 производится следующим образом:

ip domain-lookup

ip name-server 192.168.218.1

- Настройка на маршрутизаторе R1 статических маршрутов на сети из таблицы VLAN, ведущие на коммутатор ядра SW1 осуществляется следующим образом:

ip route 192.168.18.0 255.255.255.0 192.168.18.201

ip route 192.168.118.0 255.255.255.0 192.168.118.201

ip route 192.168.218.0 255.255.255.0 192.168.218.201

- Настройка на маршрутизаторе R1 трансляции адресов (NAT) для доступа в интернет IP-адресов из VLAN Internet, используя в качестве глобального адреса адрес на интерфейсе GigabitEthernet0/0, производится следующим образом:

access-list 1 permit 192.168.218.0 0.0.0.255

ip nat inside source list 1 interface gi0/0 overload

(gi0/1.218)

ip nat inside

(gi0/0)

ip nat outside

- Настройка на коммутаторе SW1 DHCP-сервер во VLAN Internet и Cash, используя пулы адресов с маской /24, маршрут по умолчанию на SW1 и DNS-сервер на R1, а также назначение исключения в пулах для статических адресов, назначенных сетевым устройствам, осуществляется следующим образом:

На R1:

Gi0/0

ip address dhcp hostname R1

На SW1:

ip dhcp excluded-address 192.168.218.1

ip dhcp excluded-address 192.168.218.201

ip dhcp excluded-address 192.168.18.201

ip dhcp excluded-address 192.168.118.201 192.168.118.205

ip dhcp excluded-address 192.168.118.2

ip dhcp excluded-address 192.168.218.2

ip dhcp excluded-address 192.168.18.2

ip dhcp pool INTERNET

network 192.168.218.0 255.255.255.0

default-router 192.168.218.201

dns-server 192.168.218.1

lease 10

ip dhcp pool CASH

network 192.168.18.0 255.255.255.0

default-router 192.168.18.201

lease 10

- Перевод портов GigabitEthernet1/0 на коммутаторах SW4 и SW5 в режим access и назначение им VLAN Internet (SW4) и Cash (SW5) соответственно производится следующим образом:

interface GigabitEthernet 1/0

switchport mode access

switchport access vlan 218 (для SW5 vlan 318)

Проверка доступа в Интернет с узла Win (Windows 10) в режиме DHCP осуществляется следующим образом:

- Настройка сервера Server под управлением Ubuntu 20.04 интерфейса eth0 в режиме trunk 802.1Q со всеми VLAN из таблицы, используя netplan и статические адреса, производится следующим образом:

network:

version: 2

renderer: networkd

ethernets:

eth0:

dhcp4: no

vlans:

MGMT:

id: 118

link: eth0

addresses: [192.168.118.2/24]

dhcp4: no

gateway4: 192.168.118.201

Internet:

id: 218

link: eth0

addresses: [192.168.218.2/24]

dhcp4: no

gateway4: 192.168.218.201

nameservers:

addresses: [192.168.218.1]

Cash:

id: 318

link: eth0

addresses: [192.168.18.2/24]

dhcp4: no

gateway4: 192.168.18.201

netplan apply

Проверка доступа в Интернет с сервера, а также доступа к серверу с узла Win и узла Linux, осуществляется следующим образом:

Также маршрутизаторы и коммутаторы в сети необходимо обеспечить возможностью удалённого подключения по ssh и запретить осуществлять через них подключение по http/https. Производится это следующим образом:

ip domain-name labaz.org

crypto key generate rsa

1024

no ip http server

no ip http secure-server

line vty 0 4

transport input ssh

login authentication default

Конфигурация на R1

no service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$.LJ5$uj9hDokwZ58CXz5E0seGr0

!

aaa new-model

!

aaa authentication login default local

!

aaa session-id common

!

mmi polling-interval 60

no mmi auto-configure

no mmi pvc

mmi snmp-timeout 180

!

no ip icmp rate-limit unreachable

!

ip domain name labaz.org

ip name-server 9.9.9.9

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

username admin privilege 15 password 0 admin18

username checker privilege 15 password 0 PfxtvXtrth!

!

redundancy

!

no cdp log mismatch duplex

!

ip tcp synwait-time 5

!

interface GigabitEthernet0/0

ip address dhcp hostname R1

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

media-type rj45

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

media-type rj45

!

interface GigabitEthernet0/1.118

encapsulation dot1Q 118

ip address 192.168.118.1 255.255.255.0

!

interface GigabitEthernet0/1.218

encapsulation dot1Q 218

ip address 192.168.218.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

media-type rj45

!

interface GigabitEthernet0/3

no ip address

shutdown

duplex auto

speed auto

media-type rj45

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

ip dns server

ip nat inside source list 1 interface GigabitEthernet0/0 overload

ip route 192.168.18.0 255.255.255.0 192.168.18.201

ip route 192.168.118.0 255.255.255.0 192.168.118.201

ip route 192.168.218.0 255.255.255.0 192.168.218.201

!

ipv6 ioam timestamp

!

access-list 1 permit 192.168.218.0 0.0.0.255

!

line con 0

exec-timeout 0 0

privilege level 15

logging synchronous

line aux 0

exec-timeout 0 0

privilege level 15

logging synchronous

line vty 0 4

transport input ssh

!

no scheduler allocate

Конфигурация на SW1

no service password-encryption

service compress-config

!

hostname SW1

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$iA71$bMp3nAdJFhtkYgv9g3AXk1

!

username admin privilege 15 password 0 admin18

username checker privilege 15 password 0 PfxtvXtrth!

aaa new-model

!

aaa authentication login default local

!

aaa session-id common

!

ip dhcp excluded-address 192.168.218.1

ip dhcp excluded-address 192.168.218.201

ip dhcp excluded-address 192.168.18.201

ip dhcp excluded-address 192.168.118.201 192.168.118.205

ip dhcp excluded-address 192.168.118.2

ip dhcp excluded-address 192.168.218.2

ip dhcp excluded-address 192.168.18.2

!

ip dhcp pool INTERNET

network 192.168.218.0 255.255.255.0

default-router 192.168.218.201

dns-server 192.168.218.1

lease 10

!

ip dhcp pool CASH

network 192.168.18.0 255.255.255.0

default-router 192.168.18.201

lease 10

!

ip domain-name labaz.org

ip name-server 192.168.218.1

ip cef

no ipv6 cef

!

spanning-tree mode pvst

spanning-tree extend system-id

spanning-tree vlan 118,218,318 priority 24576

!

interface Port-channel1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Port-channel2

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface GigabitEthernet0/0

switchport trunk allowed vlan 118,218

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 1 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 1 mode active

!

interface GigabitEthernet0/3

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet1/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 2 mode active

!

interface GigabitEthernet1/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 2 mode active

!

interface GigabitEthernet1/2

negotiation auto

!

interface GigabitEthernet1/3

negotiation auto

!

interface Vlan1

no ip address

shutdown

!

interface Vlan118

ip address 192.168.118.201 255.255.255.0

!

interface Vlan218

ip address 192.168.218.201 255.255.255.0

!

interface Vlan318

ip address 192.168.18.201 255.255.255.0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 192.168.218.1

ip route 0.0.0.0 0.0.0.0 Vlan218 192.168.218.1

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

line con 0

line aux 0

line vty 0 4

transport input ssh

Spanning tree

VLAN0118

Spanning tree enabled protocol ieee

Root ID    Priority    24694

Address     50e5.a000.0000

This bridge is the root

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    24694  (priority 24576 sys-id-ext 118)

Address     50e5.a000.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Desg FWD 4         128.1    P2p

Gi0/3               Desg FWD 4         128.4    P2p

Po1                 Desg FWD 3         128.65   P2p

Po2                 Desg FWD 3         128.66   P2p

VLAN0218

Spanning tree enabled protocol ieee

Root ID    Priority    24794

Address     50e5.a000.0000

This bridge is the root

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    24794  (priority 24576 sys-id-ext 218)

Address     50e5.a000.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Desg FWD 4         128.1    P2p

Gi0/3               Desg FWD 4         128.4    P2p

Po1                 Desg FWD 3         128.65   P2p

Po2                 Desg FWD 3         128.66   P2p

VLAN0318

Spanning tree enabled protocol ieee

Root ID    Priority    24894

Address     50e5.a000.0000

This bridge is the root

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    24894  (priority 24576 sys-id-ext 318)

Address     50e5.a000.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/3               Desg FWD 4         128.4    P2p

Po1                 Desg FWD 3         128.65   P2p

Po2                 Desg FWD 3         128.66   P2p

Конфигурация на SW2

no service password-encryption

service compress-config

!

hostname SW2

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$E7Qx$X6Z6SuF2hE6b9nJ..R8K2.

!

username admin privilege 15 password 0 admin18

username checker privilege 15 password 0 PfxtvXtrth!

aaa new-model

!

aaa authentication login default local

!

aaa session-id common

!

ip domain-name labaz.org

ip name-server 192.168.218.1

ip cef

no ipv6 cef

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Port-channel2

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface GigabitEthernet0/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 1 mode active

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 1 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 2 mode active

!

interface GigabitEthernet0/3

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 2 mode active

!

interface GigabitEthernet1/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet1/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet1/2

negotiation auto

!

interface GigabitEthernet1/3

negotiation auto

!

interface Vlan1

no ip address

shutdown

!

interface Vlan118

ip address 192.168.118.202 255.255.255.0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Vlan118 192.168.118.201

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

line con 0

line aux 0

line vty 0 4

transport input ssh

Spanning-tree

VLAN0118

Spanning tree enabled protocol ieee

Root ID    Priority    24694

Address     50e5.a000.0000

Cost        3

Port        65 (Port-channel1)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32886  (priority 32768 sys-id-ext 118)

Address     5077.7100.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi1/0               Desg FWD 4         128.5    P2p

Gi1/1               Desg FWD 4         128.6    P2p

Po1                 Root FWD 3         128.65   P2p

Po2                 Desg FWD 3         128.66   P2p

VLAN0218

Spanning tree enabled protocol ieee

Root ID    Priority    24794

Address     50e5.a000.0000

Cost        3

Port        65 (Port-channel1)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32986  (priority 32768 sys-id-ext 218)

Address     5077.7100.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi1/0               Desg FWD 4         128.5    P2p

Gi1/1               Desg FWD 4         128.6    P2p

Po1                 Root FWD 3         128.65   P2p

Po2                 Desg FWD 3         128.66   P2p

VLAN0318

Spanning tree enabled protocol ieee

Root ID    Priority    24894

Address     50e5.a000.0000

Cost        3

Port        65 (Port-channel1)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    33086  (priority 32768 sys-id-ext 318)

Address     5077.7100.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi1/0               Desg FWD 4         128.5    P2p

Gi1/1               Desg FWD 4         128.6    P2p

Po1                 Root FWD 3         128.65   P2p

Po2                 Desg FWD 3         128.66   P2p

Конфигурация на SW3

no service password-encryption

service compress-config

!

hostname SW3

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$eqww$1yoUe9W.YE9o7UTKZJw6F/

!

username admin privilege 15 password 0 admin18

username checker privilege 15 password 0 PfxtvXtrth!

aaa new-model

!

aaa authentication login default local

!

aaa session-id common

!

ip domain-name labaz.org

ip cef

no ipv6 cef

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Port-channel2

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface GigabitEthernet0/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 1 mode active

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 1 mode active

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 2 mode active

!

interface GigabitEthernet0/3

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

channel-group 2 mode active

!

interface GigabitEthernet1/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet1/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet1/2

negotiation auto

!

interface GigabitEthernet1/3

negotiation auto

!

interface Vlan118

ip address 192.168.118.203 255.255.255.0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Vlan118 192.168.118.201

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

line con 0

line aux 0

line vty 0 4

transport input ssh

Spanning-tree

VLAN0118

Spanning tree enabled protocol ieee

Root ID    Priority    24694

Address     50e5.a000.0000

Cost        3

Port        65 (Port-channel1)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32886  (priority 32768 sys-id-ext 118)

Address     50c8.f700.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi1/0               Desg FWD 4         128.5    P2p

Gi1/1               Desg FWD 4         128.6    P2p

Po1                 Root FWD 3         128.65   P2p

Po2                 Altn BLK 3         128.66   P2p

VLAN0218

Spanning tree enabled protocol ieee

Root ID    Priority    24794

Address     50e5.a000.0000

Cost        3

Port        65 (Port-channel1)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32986  (priority 32768 sys-id-ext 218)

Address     50c8.f700.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi1/0               Desg FWD 4         128.5    P2p

Gi1/1               Desg FWD 4         128.6    P2p

Po1                 Root FWD 3         128.65   P2p

Po2                 Altn BLK 3         128.66   P2p

VLAN0318

Spanning tree enabled protocol ieee

Root ID    Priority    24894

Address     50e5.a000.0000

Cost        3

Port        65 (Port-channel1)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    33086  (priority 32768 sys-id-ext 318)

Address     50c8.f700.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi1/0               Desg FWD 4         128.5    P2p

Gi1/1               Desg FWD 4         128.6    P2p

Po1                 Root FWD 3         128.65   P2p

Po2                 Altn BLK 3         128.66   P2p

Конфигурация на SW4

no service password-encryption

service compress-config

!

hostname SW4

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$7jX4$9L7yeyRbgoz/B0/tNWrhL0

!

username admin privilege 15 password 0 admin18

username checker privilege 15 password 0 PfxtvXtrth!

aaa new-model

!

aaa authentication login default local

!

aaa session-id common

!

ip domain-name labaz.org

ip name-server 192.168.218.1

ip cef

no ipv6 cef

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface GigabitEthernet0/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

spanning-tree cost 3

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet0/2

negotiation auto

!

interface GigabitEthernet0/3

negotiation auto

!

interface GigabitEthernet1/0

switchport access vlan 218

switchport mode access

negotiation auto

!

interface GigabitEthernet1/1

negotiation auto

!

interface GigabitEthernet1/2

negotiation auto

!

interface GigabitEthernet1/3

negotiation auto

!

interface Vlan118

ip address 192.168.118.204 255.255.255.0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Vlan118 192.168.118.201

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

line con 0

line aux 0

line vty 0 4

transport input ssh

Spanning-tree

VLAN0118

Spanning tree enabled protocol ieee

Root ID    Priority    24694

Address     50e5.a000.0000

Cost        6

Port        1 (GigabitEthernet0/0)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32886  (priority 32768 sys-id-ext 118)

Address     50b0.3a00.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Root FWD 3         128.1    P2p

Gi0/1               Altn BLK 4         128.2    P2p

VLAN0218

Spanning tree enabled protocol ieee

Root ID    Priority    24794

Address     50e5.a000.0000

Cost        6

Port        1 (GigabitEthernet0/0)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32986  (priority 32768 sys-id-ext 218)

Address     50b0.3a00.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Root FWD 3         128.1    P2p

Gi0/1               Altn BLK 4         128.2    P2p

Gi1/0               Desg FWD 4         128.5    P2p

VLAN0318

Spanning tree enabled protocol ieee

Root ID    Priority    24894

Address     50e5.a000.0000

Cost        6

Port        1 (GigabitEthernet0/0)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    33086  (priority 32768 sys-id-ext 318)

Address     50b0.3a00.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Root FWD 3         128.1    P2p

Gi0/1               Altn BLK 4         128.2    P2p

Конфигурация на SW5

no service password-encryption

service compress-config

!

hostname SW5

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$d7os$keEcLR1rnMbDn54xGC2Pl.

!

username admin privilege 15 password 0 admin18

username checker privilege 15 password 0 PfxtvXtrth!

aaa new-model

!

aaa authentication login default local

!

aaa session-id common

!

ip domain-name labaz.org

ip name-server 192.168.218.1

ip cef

no ipv6 cef

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface GigabitEthernet0/0

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

spanning-tree cost 3

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 118,218,318

switchport trunk encapsulation dot1q

switchport mode trunk

negotiation auto

!

interface GigabitEthernet0/2

negotiation auto

!

interface GigabitEthernet0/3

negotiation auto

!

interface GigabitEthernet1/0

switchport access vlan 318

switchport mode access

negotiation auto

!

interface GigabitEthernet1/1

negotiation auto

!

interface GigabitEthernet1/2

negotiation auto

!

interface GigabitEthernet1/3

negotiation auto

!

interface Vlan118

ip address 192.168.118.205 255.255.255.0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Vlan118 192.168.118.201

ip ssh server algorithm encryption aes128-ctr aes192-ctr aes256-ctr

ip ssh client algorithm encryption aes128-ctr aes192-ctr aes256-ctr

!

line con 0

line aux 0

line vty 0 4

transport input ssh

Spanning-tree

VLAN0118

Spanning tree enabled protocol ieee

Root ID    Priority    24694

Address     50e5.a000.0000

Cost        6

Port        1 (GigabitEthernet0/0)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32886  (priority 32768 sys-id-ext 118)

Address     505e.df00.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Root FWD 3         128.1    P2p

Gi0/1               Altn BLK 4         128.2    P2p

VLAN0218

Spanning tree enabled protocol ieee

Root ID    Priority    24794

Address     50e5.a000.0000

Cost        6

Port        1 (GigabitEthernet0/0)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32986  (priority 32768 sys-id-ext 218)

Address     505e.df00.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Root FWD 3         128.1    P2p

Gi0/1               Altn BLK 4         128.2    P2p

VLAN0318

Spanning tree enabled protocol ieee

Root ID    Priority    24894

Address     50e5.a000.0000

Cost        6

Port        1 (GigabitEthernet0/0)

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    33086  (priority 32768 sys-id-ext 318)

Address     505e.df00.0000

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Aging Time  300 sec

Interface           Role Sts Cost      Prio.Nbr Type

------------------- ---- --- --------- -------- --------------------------------

Gi0/0               Root FWD 3         128.1    P2p

Gi0/1               Altn BLK 4         128.2    P2p

Gi1/0               Desg FWD 4         128.5    P2p

Выводы

В процессе выполнения данной лабораторной работы я научился настраивать взаимодействие между устройствами в небольшой сети, познакомился с подключением устройств через разные VLANы. Также в данной работе я впервые взаимодействовал с сервером, что также было познавательно. Теперь мне стало намного понятнее, каким образом устройства в сети “общаются” между собой.